import pandas as pd

import scipy.signal as scisig

import os

import numpy as np

def get_user_input(prompt):

try:

return raw_input(prompt)

except NameError:

return input(prompt)

def getInputLoadFile():

'''Asks user for type of file and file path. Loads corresponding data.

OUTPUT:

data:   DataFrame, index is a list of timestamps at 8Hz, columns include

AccelZ, AccelY, AccelX, Temp, EDA, filtered_eda

'''

print("Please enter information about your EDA file... ")

dataType = get_user_input("\tData Type (e4, q, shimmer, or misc): ")

if dataType=='q':

filepath = get_user_input("\tFile path: ")

filepath_confirm = filepath

data = loadData_Qsensor(filepath)

elif dataType=='e4':

filepath = get_user_input("\tPath to E4 directory: ")

filepath_confirm = os.path.join(filepath,"EDA.csv")

data = loadData_E4(filepath)

elif dataType=='shimmer':

filepath = get_user_input("\tFile path: ")

filepath_confirm = filepath

data = loadData_shimmer(filepath)

elif dataType=="misc":

filepath = get_user_input("\tFile path: ")

filepath_confirm = filepath

data = loadData_misc(filepath)

else:

print("Error: not a valid file choice")

return data, filepath_confirm

def getOutputPath():

print("")

print("Where would you like to save the computed output file?")

outfile = get_user_input('\tFile name: ')

outputPath = get_user_input('\tFile directory (./ for this directory): ')

fullOutputPath = os.path.join(outputPath,outfile)

if fullOutputPath[-4:] != '.csv':

fullOutputPath = fullOutputPath+'.csv'

return fullOutputPath

def loadData_Qsensor(filepath):

'''

This function loads the Q sensor data, uses a lowpass butterworth filter on the EDA signal

Note: currently assumes sampling rate of 8hz, 16hz, 32hz; if sampling rate is 16hz or 32hz the signal is downsampled

INPUT:

filepath:       string, path to input file

OUTPUT:

data:           DataFrame, index is a list of timestamps at 8Hz, columns include AccelZ, AccelY, AccelX, Temp, EDA, filtered_eda

'''

# Get header info

try:

header_info = pd.io.parsers.read_csv(filepath, nrows=5)

except IOError:

print("Error!! Couldn't load file, make sure the filepath is correct and you are using a csv from the q sensor software\n\n")

return

# Get sample rate

sampleRate = int((header_info.iloc[3,0]).split(":")[1].strip())

# Get the raw data

data = pd.io.parsers.read_csv(filepath, skiprows=7)

data = data.reset_index()

# Reset the index to be a time and reset the column headers

data.columns = ['AccelZ','AccelY','AccelX','Battery','Temp','EDA']

# Get Start Time

startTime = pd.to_datetime(header_info.iloc[4,0][12:-10])

# Make sure data has a sample rate of 8Hz

data = interpolateDataTo8Hz(data,sampleRate,startTime)

# Remove Battery Column

data = data[['AccelZ','AccelY','AccelX','Temp','EDA']]

# Get the filtered data using a low-pass butterworth filter (cutoff:1hz, fs:8hz, order:6)

data['filtered_eda'] =  butter_lowpass_filter(data['EDA'], 1.0, 8, 6)

return data

def _loadSingleFile_E4(filepath,list_of_columns, expected_sample_rate,freq):

# Load data

data = pd.read_csv(filepath)

# Get the startTime and sample rate

startTime = pd.to_datetime(float(data.columns.values[0]),unit="s")

sampleRate = float(data.iloc[0][0])

data = data[data.index!=0]

data.index = data.index-1

# Reset the data frame assuming expected_sample_rate

data.columns = list_of_columns

if sampleRate != expected_sample_rate:

print('ERROR, NOT SAMPLED AT {0}HZ. PROBLEMS WILL OCCUR\n'.format(expected_sample_rate))

# Make sure data has a sample rate of 8Hz

data = interpolateDataTo8Hz(data,sampleRate,startTime)

return data

def loadData_E4(filepath):

# Load EDA data

eda_data = _loadSingleFile_E4(os.path.join(filepath,'EDA.csv'),["EDA"],4,"250L")

# Get the filtered data using a low-pass butterworth filter (cutoff:1hz, fs:8hz, order:6)

eda_data['filtered_eda'] =  butter_lowpass_filter(eda_data['EDA'], 1.0, 8, 6)

# Load ACC data

acc_data = _loadSingleFile_E4(os.path.join(filepath,'ACC.csv'),["AccelX","AccelY","AccelZ"],32,"31250U")

# Scale the accelometer to +-2g

acc_data[["AccelX","AccelY","AccelZ"]] = acc_data[["AccelX","AccelY","AccelZ"]]/64.0

# Load Temperature data

temperature_data = _loadSingleFile_E4(os.path.join(filepath,'TEMP.csv'),["Temp"],4,"250L")

data = eda_data.join(acc_data, how='outer')

data = data.join(temperature_data, how='outer')

# E4 sometimes records different length files - adjust as necessary

min_length = min(len(acc_data), len(eda_data), len(temperature_data))

return data[:min_length]

def loadData_shimmer(filepath):

data = pd.read_csv(filepath, sep='\t', skiprows=(0,1))

orig_cols = data.columns

rename_cols = {}

for search, new_col in [['Timestamp','Timestamp'],

['Accel_LN_X', 'AccelX'], ['Accel_LN_Y', 'AccelY'], ['Accel_LN_Z', 'AccelZ'],

['Skin_Conductance', 'EDA']]:

orig = [c for c in orig_cols if search in c]

if len(orig) == 0:

continue

rename_cols[orig[0]] = new_col

data.rename(columns=rename_cols, inplace=True)

# TODO: Assuming no temperature is recorded

data['Temp'] = 0

# Drop the units row and unnecessary columns

data = data[data['Timestamp'] != 'ms']

data.index = pd.to_datetime(data['Timestamp'], unit='ms')

data = data[['AccelZ', 'AccelY', 'AccelX', 'Temp', 'EDA']]

for c in ['AccelZ', 'AccelY', 'AccelX', 'Temp', 'EDA']:

data[c] = pd.to_numeric(data[c])

# Convert to 8Hz

data = data.resample("125L").mean()

data.interpolate(inplace=True)

# Get the filtered data using a low-pass butterworth filter (cutoff:1hz, fs:8hz, order:6)

data['filtered_eda'] = butter_lowpass_filter(data['EDA'], 1.0, 8, 6)

return data

def loadData_getColNames(data_columns):

print("Here are the data columns of your file: ")

print(data_columns)

# Find the column names for each of the 5 data streams

colnames = ['EDA data','Temperature data','Acceleration X','Acceleration Y','Acceleration Z']

new_colnames = ['','','','','']

for i in range(len(new_colnames)):

new_colnames[i] = get_user_input("Column name that contains "+colnames[i]+": ")

while (new_colnames[i] not in data_columns):

print("Column not found. Please try again")

print("Here are the data columns of your file: ")

print(data_columns)

new_colnames[i] = get_user_input("Column name that contains "+colnames[i]+": ")

# Get user input on sample rate

sampleRate = get_user_input("Enter sample rate (must be an integer power of 2): ")

while (sampleRate.isdigit()==False) or (np.log(int(sampleRate))/np.log(2) != np.floor(np.log(int(sampleRate))/np.log(2))):

print("Not an integer power of two")

sampleRate = get_user_input("Enter sample rate (must be a integer power of 2): ")

sampleRate = int(sampleRate)

# Get user input on start time

startTime = pd.to_datetime(get_user_input("Enter a start time (format: YYYY-MM-DD HH:MM:SS): "))

while type(startTime)==str:

print("Not a valid date/time")

startTime = pd.to_datetime(get_user_input("Enter a start time (format: YYYY-MM-DD HH:MM:SS): "))

return sampleRate, startTime, new_colnames

def loadData_misc(filepath):

# Load data

data = pd.read_csv(filepath)

# Get the correct colnames

sampleRate, startTime, new_colnames = loadData_getColNames(data.columns.values)

data.rename(columns=dict(zip(new_colnames,['EDA','Temp','AccelX','AccelY','AccelZ'])), inplace=True)

data = data[['AccelZ','AccelY','AccelX','Temp','EDA']]

# Make sure data has a sample rate of 8Hz

data = interpolateDataTo8Hz(data,sampleRate,startTime)

# Get the filtered data using a low-pass butterworth filter (cutoff:1hz, fs:8hz, order:6)

data['filtered_eda'] =  butter_lowpass_filter(data['EDA'], 1.0, 8, 6)

return data

def interpolateDataTo8Hz(data,sample_rate,startTime):

if sample_rate<8:

# Upsample by linear interpolation

if sample_rate==2:

data.index = pd.date_range(start=startTime, periods=len(data), freq='500L')

elif sample_rate==4:

data.index = pd.date_range(start=startTime, periods=len(data), freq='250L')

data = data.resample("125L").mean()

else:

if sample_rate>8:

# Downsample

idx_range = list(range(0,len(data))) # TODO: double check this one

data = data.iloc[idx_range[0::int(int(sample_rate)/8)]]

# Set the index to be 8Hz

data.index = pd.date_range(start=startTime, periods=len(data), freq='125L')

# Interpolate all empty values

data = interpolateEmptyValues(data)

return data

def interpolateEmptyValues(data):

cols = data.columns.values

for c in cols:

data.loc[:, c] = data[c].interpolate()

return data

def butter_lowpass(cutoff, fs, order=5):

# Filtering Helper functions

nyq = 0.5 * fs

normal_cutoff = cutoff / nyq

b, a = scisig.butter(order, normal_cutoff, btype='low', analog=False)

return b, a

def butter_lowpass_filter(data, cutoff, fs, order=5):

# Filtering Helper functions

b, a = butter_lowpass(cutoff, fs, order=order)

y = scisig.lfilter(b, a, data)

return y